Enterprise Networking Project  / Simple Networking Project

Design a network in CISCO packet tracer to connect ACCOUNTS and  DELIVERY departments through the following:

- Each department should contain at least 2 PCs

- Appropriate number of switches and routers should be used in the network

- Using the given network address 192.168.40.0, all interfaces should be configured with appropriate IP address, subnet mask and gateway

- All devices in the network should be connected using appropriate cables

- Test the connectivity between ACCOUNTS and  DELIVERY department PCs in DELIVERY department should be able to ping the PCs in ACCOUNTS department.

Topology

Subnetting Network

- Network address = 192.168.40.0

- Number of network = 2

2^n=# of subnets ===== 2^n=2

N=1

255.255.255.255

11111111.11111111.11111111.10000000 -----Binary

255.255.255.128subnet mask

1st Subnet

Subnet mask = 255.255.255.128

Network ID = 192.168.40.0

Range of Vlid Host = 192.168.40.1 – 192.168.40.126

Broadcast ID = 192.168.40.127

2st Subnet

Subnet mask = 255.255.255.128

Network ID = 192.168.40.128

Range of Vlid Host = 192.168.40.129 – 192.168.40.254

Broadcast ID = 192.168.40.255

IP address

ACCOUNTS

R1

R1(config)#int g0/0

R1(config-if)#ip add 192.168.40.1 255.255.255.128

R1(config-if)#no shut

PC0 – 192.168.40.2 255.255.255.128 192.168.40.1

PC1 – 192.168.40.3 255.255.255.128 192.168.40.1

Printer – 192.168.40.4 255.255.255.128 192.168.40.1

DELIVERY

R1

R1(config-if)#int g0/1

R1(config-if)#ip add 192.168.40.129 255.255.255.128

R1(config-if)#no shut

PC2 – 192.168.40.130 255.255.255.128 192.168.40.129

PC3 – 192.168.40.131 255.255.255.128 192.168.40.129

Printer – 192.168.40.132 255.255.255.128 192.168.40.129