Enterprise Networking | Small Office Home Office

XYZ company is a fast-growing company in Eastern Australia with more than 2 million customers globally. The company deals with selling and buying of food items, which are basically operated from the headquarters. The company is intending to open a branch near the local village Bonalbo. Thus, the company requires young IT graduates to design the network for the branch. The network is intended to operate separately from the HQ network.

Being a small network, the company has the following requirements during implementation.

- One router and one switch to be used

- 3 departments (Admin/IT, Finance/HR and Customer Service/Reception)

- Each department is required to be in different VLANS

- Each department is required to have wireless network for the users

- Host devices in the network are required to obtain IPv4 address automatically

- Devices in all the departments are required to communicate with each other

Assume the ISP gave out a base network 192.168.1.0, you as the young network engineer who has been hired, design and implement a network considering the above requirements.

Topology

Subnetting

Base Network: 192.168.1.0

No. of subnets = 3

Formula : No. of subnets = 2^n (n represents number of borrows bits)

2^n=3 =====  n=2

Class C = 255.255.255.0 === Binary 11111111. 11111111. 11111111.00000000

Borrow 2 bits ==== 11111111. 11111111. 11111111.11000000 == 255.255.255.192

| 1st Subnet Net ID: 192.168.1.0 Broadcast ID: 192.168.1.63 Host Range: 192.168.1.1-192.168.1.62 | 3rd Subnet Net ID: 192.168.1.128 Broadcast ID: 192.168.191 Host Range: 192.168.1.129-192.168.1.190 |
| --- | --- |
| 2nd Subnet Net ID: 192.168.1.64 Broadcast ID: 192.168.1.127 Host Range: 192.168.1.65 – 192.168.1.126 | 4th Subnet Net ID:192.168.1.192 Broadcast ID: 192.168.1.255 Host Range: 192.168.1.193- 192.168.1.254 |

IP Addresses:

R1

------------------------------------Bring interface UP

Router(config)#hostname R1

R1(config)#int g0/0

R1(config-if)#no sh

------------------------------------Inter-vlan Routing

Admin/IT

R1(config-if)#int g0/0.10

R1(config-subif)#encapsulation dot1Q 10

R1(config-subif)#ip address 192.168.1.1 255.255.255.192

Finance/HR

R1(config-if)#int g0/0.20

R1(config-subif)#encapsulation dot1Q 20

R1(config-subif)#ip address 192.168.1.65 255.255.255.192

CS/Reception

R1(config-if)#int g0/0.10

R1(config-subif)#encapsulation dot1Q 30

R1(config-subif)#ip address 192.168.1.129 255.255.255.192

------------------------------------Config DHCP

R1(config)#service dhcp

------------------------------------Config DHCP Pools, gateways, dns & domain

R1(config)#ip dhcp pool Admin-Pool

R1(dhcp-config)#network 192.168.1.0 255.255.255.192

R1(dhcp-config)#default-router 192.168.1.1

R1(dhcp-config)#dns-server 192.168.1.1

R1(dhcp-config)#domain-name Admin.com

R1(config)#ip dhcp pool Finance-Pool

R1(dhcp-config)#network 192.168.1.64 255.255.255.192

R1(dhcp-config)#default-router 192.168.1.65

R1(dhcp-config)#dns-server 192.168.1.65

R1(dhcp-config)#domain-name Finance.com

R1(config)#ip dhcp pool CS-Pool

R1(dhcp-config)#network 192.168.1.28 255.255.255.192

R1(dhcp-config)#network 192.1 68.1.128 255.255.255.192

R1(dhcp-config)#default-router 192.168.1.129

R1(dhcp-config)#dns-server 192.168.1.129

R1(dhcp-config)#domain-name CS.com

Sw1

------------------------------------VLAN

SW1(config)#vlan 10

SW1(config-vlan)#name Admin/IT

SW1(config-vlan)#vlan 20

SW1(config-vlan)#name Finance/HR

SW1(config-vlan)#vlan 30

SW1(config-vlan)#name CS/Reception

------------------------------------Port Access to vlans per department

Admin/IT

SW1(config)#int ran f0/2-4

SW1(config-if-range)#switchport mode access

SW1(config-if-range)#switchport access vlan 10

Finance/HR

SW1(config)#int rang f0/5-7

SW1(config-if-range)#switchport mode access

SW1(config-if-range)#switchport access vlan 20

CS/Reception

SW1(config)#int ran fa0/8-10

SW1(config-if-range)#switchport mode access

SW1(config-if-range)#switchport access vlan 30

------------------------------------Trunking

SW1(config)#int fa0/1

SW1(config-if)#switchport mode trunk

Admin/IT VLAN 10

AP: Admin-WiFi | Admin@123

Finance/HR VLAN 20

AP: Finance-WiFi | Finance@123

CS/Reception VLAN 30

AP: CS-WiFi | Customer@123